Texas Department of Housing & Community Affairs

Emergency Solutions Shelter/Facility Inspection Checklist

i

| Site contact: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Site Contact Printed Name Date |  |  |  |  |  |  |
| Inspector: |  |  |  |  |  |  |
|  | Signature |  | Printed Name |  | Date |  |

A. GENERAL INFORMATION

Funded Site

| Name: |  |
| --- | --- |
| TDHCA # |  |
| Address: |  |
| City / Zip Code: |  |
| Site Contact ph# |  |

TDHCA Contact Info  (shown in CMTS)

| Contract Entity: |  |
| --- | --- |
| Contract Contact: |  |
| Contract Address: |  |
| City / State Zip |  |
| Owner Contact # |  |

| B. FUNDING INFORMATION |  |
| --- | --- |
| Funded Activities: | 1. |
|  | 2. |
|  | 3. |

ESG funded operation of short-term emergency shelters and the delivery of essential services is governed by Subpart K of the Lead-Based Paint Hazard regulations. According to the Interpretive Guidance to the regulations, link to Interpretive Guidance web page (http://www.hud.gov/lea/1012QA_final_sept21.PDF), most emergency shelters are exempt from the lead-based paint regulations.

| C. LEAD BASED PAINT |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A or Inc. (Provide Explanation Below) |
| 24 CFR 576.403 | Was Date of Original Construction Before January 1, 1978? |  |  |  |
| 24 CFR 576.403 | Does the shelter offer longer-term transitional housing in an apartment with one or more bedrooms AND which has family residents who are part of a program requiring continual residence of more than 100 days. If yes, complete ESG Permanent Housing Checklist for each inspected unit. |  |  |  |

| D. HABITABILITY |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A or Inc. (Provide Explanation Below) |
| 24 CFR 576.403 | (1) a. Structure and materials. The shelter building must be structurally sound to protect residents from the elements and not pose any threat to health and safety of the residents. Does the building appear structurally sound and free from any threat to health and safety? |  |  |  |
| 24 CFR 576.403 | (1) b. Structure and materials. Any renovation (including major rehabilitation and conversion) carried out with ESG assistance must use Energy Star and Water Sense products and appliances. Do all newly installed products meet these requirements? |  |  |  |
| 24 CFR 576.403 | (2) a. Access. The shelter must be accessible in accordance with Section 504 of the Rehabilitation Act (29 U.S.C. 794) and implementing regulations at 24 CFR part 8; the Fair Housing Act (42 U.S.C. 3601 et seq.) and implementing regulations at 24 CFR part 100; and Title II of the Americans with Disabilities Act(42 U.S.C. 12131 et seq.) and 28 CFR part 35; where applicable. |  |  |  |
| 24 CFR 576.403 | (2) b. Access Housing must be accessible and capable of being utilized without unauthorized use of private properties. |  |  |  |
| 24 CFR 576.403 | (3) Space and security. Except where the shelter is intended for day use only, the shelter must provide each program participant in the shelter with an acceptable place to sleep and adequate space and security for themselves and their belongings. Does the sleeping area meet these requirements? |  |  |  |
| 24 CFR 576.403 | (4) Interior air quality. Each room or space within the shelter must have a natural or mechanical means of ventilation. The interior air must be free of pollutants at a level that might threaten or harm the health of residents. Does the facility meet these requirements? |  |  |  |

| D. HABITABILITY (cont.) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A or Inc. (Provide Explanation Below) |
| 24 CFR 576.403 | (5) Water supply. The shelter’s water supply must be free of contamination. Does the water supply appear safe for drinking? |  |  |  |
| 24 CFR 576.403 | (6) Sanitary facilities. Each program participant in the shelter must have access to sanitary facilities that are in proper operating condition, are private, and are adequate for personal cleanliness and the disposal of human waste. Do the restrooms meet these requirements? |  |  |  |
| 24 CFR 576.403 | (7) Thermal environment. The shelter must have any necessary heating/ cooling facilities in proper operating condition. Does the facility meet these requirements? |  |  |  |
| 24 CFR 576.403 | (8) Illumination and electricity. The shelter must have adequate natural or artificial illumination to permit normal indoor activities and support health and safety. There must be sufficient electrical sources to permit the safe use of electrical appliances in the shelter. Does the facility meet these requirements? |  |  |  |
| 24 CFR 576.403 | (9) Food preparation. Food preparation areas, if any, must contain suitable space and equipment to store, prepare, and serve food in a safe and sanitary manner. Does the kitchen, if any, meet these requirements? |  |  |  |
| 24 CFR 576.403 | (10) Sanitary conditions. The shelter must be maintained in a sanitary condition. Is the shelter generally clean and sanitary? |  |  |  |
| 24 CFR 576.403 | (11) a. Fire safety. There must be at least one working smoke detector in each occupied unit of the shelter. Where possible, smoke detectors must be located near sleeping areas. Does each occupied unit or sleeping area meet this requirement? |  |  |  |
| 24 CFR 576.403 | (11) b. Fire safety. All public areas of the shelter must have at least one working smoke detector. Does each public area meet this requirement? |  |  |  |
| 24 CFR 576.403 | (11) c. Fire safety. The fire alarm system must be designed for hearing-impaired residents. Does the fire alarm system meet this requirement? |  |  |  |
| 24 CFR 576.403 | (11) d. Fire safety. There must also be a second means of exiting the building in the event of fire or other emergency. Is there a second means of egress in each area of the building? |  |  |  |

| Habitability Comments and Observations |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item # |  | Actual Measurement Location/Issue |  |  |  |  | Comments (N/A requires comment) |  |

| E. ACCESSIBILITY – Common Use Facilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| “No” items must have photos and actual measurements in comments. “Inc.” means incomplete. “N/A” answers require reason why the item is not applicable noted in the comments | Building: ______ |  |  |  |  | Building: _______ |  |  |  |
| Common Use Passage Doors* on an Accessible Route | Yes | Inc | No | N/A |  | Yes | Inc | No | N/A |

| 2010 ADA 303.1 | (1) Combined heights of threshold plus any floor level change are max. ½” for swinging doors, max. ¾” for sliding doors (measured from top of threshold or track to interior floor and exterior landing), are beveled (max. 1:2 slope) if edges over ¼” high. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 2010 ADA 404.2.3 | (2) Min. 32” clear opening when door is open 90° (from door face to door stop). |  |  |  |  |
| 2010 ADA 404.2.4 | (3) Maneuvering spaces on both sides of doors are clear & level (w/max. 2% slope) w/ min. clearance. |  |  |  |  |
| 2010 ADA 404.2.7 | (4) Door handles are operable with a shape easy to grip with one hand w/out tight grasping, pinching, or twisting (e.g. lever operated, push type, and U-shaped handles). |  |  |  |  |

| Reach Range for Wall-Mounted Controls, Outlets in Common Facilities If there are multiple controls/outlets in an area, min. 1 is accessible in each | Building: | ______ |  | Building: | _______ |
| --- | --- | --- | --- | --- | --- |

| 2010 ADA 308.2 | (5) Forward reach only is possible, not over an obstruction, outlet, switch, control is in range from 15”- 48” high.. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 2010 ADA 308.3 | (6) Side reach not over obstruction, outlet/switch is in a range from 9”- 54” high. |  |  |  |  |
| 2010 ADA 308.1 | (7) Forward reach over obstruction has an open knee space and is: a) max. 20” horizontally and max. 48” above floor, or b) max. 25” horizontally and max. 44” high. |  |  |  |  |
| 2010 ADA 308.1 | (8) Side reach over a max. 34” high obstruction is max 24” horizontally and max. 46” above floor (Fig. 22). |  |  |  |  |

| E. (cont’d) Accessibility – Common Use Facilities |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Common Use Kitchen- (Kitchens that are not for common use should be reviewed as work areas) Include all other common use sinks if any (i.e. child care center, arts & crafts room) | #1: | ______ |  | #2: | ______ |

| 2010 ADA 308.2 | (9) Kitchen has a min. 60” dia. turning circle (or T-turn) or min. 40” clearance in a kitchen with a pass through design. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 2010 ADA 308.3 | (10) Accessible sink (section of counter) is min. 30” wide and max. 34” high at rim. |  |  |  |  |
| 2010 ADA 308.1 | (11) Accessible sink faucet is operable w/ one hand, without tight grasping, pinching, twisting of the wrist. |  |  |  |  |
| 2010 ADA 308.1 | (12) Clearance below accessible sink is open for knee/leg/toe room min. 30” wide, 27” high, 8” deep at top, 11” deep at 9” above floor, 17” deep & 9” high at toe (Fig 20) |  |  |  |  |
| 2010 ADA 606.5 | (13) Hot water and drain pipes under sink are fully insulated or covered w/ protection panel. There are no sharp or abrasive surfaces. |  |  |  |  |
| 2010 ADA 804.6.1 | (14) Clear floor space at sink and appliances is min. 30” by 48”. The space is perpendicular at the sink for a forward approach (19” may extend underneath). |  |  |  |  |
| 2010 ADA 226.1 | (15) Eating areas –In each eating area if tables with fixed seats are provided, do at least 5% of each table with fixed seats have accessible locations with knee space at least 27”high and 30”deep with a table top 28-34” from the floor or if movable tables are used is there at least 5% of the tables available on an accessible route. |  |  |  |  |
|  | (15) continued - or if movable tables are used is there at least 5% of the tables available on an accessible route. |  |  |  |  |

| “No” items Must have photos. Give actual measurements in comments. “Inc.” means incomplete. “N/A” answers require note in the Comments describing the reason the item is not applicable | Restroom |  | # ______ |  |  | Restroom |  | # ______ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Common Use Restroom Only one of each is needed in multiple-lavatory and multiple-commode restrooms | Yes | Inc | No | N/A |  | Yes | Inc | No | N/A |

| 2010 ADA 606.1 | (16) Accessible lavatory height is max. 34” at the rim or counter surface. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2010 ADA 309.4 | (17) Accessible lavatory faucets, dispensers and operating mechanisms are operable w/ one hand, without tight grasping, pinching, twisting of the wrist (lever operated or push type faucets). |  |  |  |  |  |  |  |  |  |
| 2010 ADA 306.3.1-5 | (18) Clearance below accessible lavatory is open for knee/leg/toe rm. min. 30” wide, 27” high, 8” deep at top, 11” deep at 9” above floor, 17” deep & 9” high at toe. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 606.5 | (19) Hot water & drain pipes under accessible lavatory are fully insulated or covered w/ protection panel. There are no sharp or abrasive surfaces. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 604.8 | (20) Accessibility within the toilet rooms-Is there an area within the toilet room with either a minimum of 60” diameter or a “T” shaped turning area. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 603.3 | (21) Mirror height is max. 40” to bottom edge of the reflecting surface (min. one if multiple provided). |  |  |  |  |  |  |  |  |  |
| 2010 ADA 604.4 | (22) Commode height is between 17” to 19” measured to the top of the seat. |  |  |  |  |  |  |  |  |  |
| v2010 ADA 604.2 | (23) Water closet shall be positioned with a wall or partition to the rear and to one side. The centerline of the water closet shall be 16 inches minimum to 18 inches maximum from the side wall or partition, except that the water closet shall be 17 inches minimum and 19 inches maximum from the side wall or partition in the ambulatory accessible toilet compartment specified in 604.8.2. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 604.6 | (24) Flush valve is the flush valve located on the wide side of the adjacent to the lavatory or is an automatic flush valve provided? |  |  |  |  |  |  |  |  |  |
| 2010 ADA 604.3.1 | (25) Commode clearance non-stall – Wheelchair accessible compartments shall be 60 inches wide minimum measured perpendicular to the side wall, and 56 inches deep minimum for wall hung water closets and 59 inches deep minimum for floor mounted water closets measured perpendicular to the rear wall. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 604.3.1 | (26) Standard stall – is there at least one stall 60” wide min. by min. 56” deep if wall-mounted commode or 59” deep if floor-mounted; exactly 18” from toilet center to one sidewall w/ side grab bar on this side; min. 32” door clearance . |  |  |  |  |  |  |  |  |  |
| 2010 ADA 4.17.3; 604.8.1.2 | (27) Standard stall door – Does the door to the stall open out (unless at the end of the row)? |  |  |  |  |  |  |  |  |  |
| 2010 ADA 213.3.1 | (28) More than six stalls– if there are more than 6 stalls is there at least one stall 60” wide min. by min. 56” deep if wall-mounted commode or 59” deep if floor-mounted; plus an additional stall at least 36” wide with a swing out door and included grab bars? |  |  |  |  |  |  |  |  |  |
| 2010 ADA 609.4 | (29) Grab bars are provided around accessible commodes mounted at 33-36” high to bar center; rear bar extends from max. 6” to min. 42” from the corner; side bar extends from max. 12” to min. 54” from the corner. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 604.3.1 | (30) Restroom doors do not swing into the 30” x 48” clear floor space required for the sink or urinal, or commode space per type (above). |  |  |  |  |  |  |  |  |  |

| E. (cont’d) Accessibility – Common Use Facilities |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| “No” items Must have photos. Give actual measurements in comments. “Inc.” means incomplete. “N/A” answers require note in the Comments describing the reason the item is not applicable |  |  |  | Restroom |  | # _____ |  |  | Restroom |  | # _____ |  |
| 2010 ADA 607.2 | (31) Clearance in front of bathtubs shall extend the length of the bathtub and shall be 30 inches wide minimum. A lavatory complying with 606 shall be permitted at the control end of the clearance. Where a permanent seat is provided at the head end of the bathtub, the clearance shall extend 12 inches minimum beyond the wall at the head end of the bathtub. Does the bathtub have proper clearance? |  |  |  |  |  |  |  |  |  |  |  |
| 2010 ADA 607.2 | (32) Seat. A permanent seat at the head end of the bathtub or a removable in-tub seat shall be provided. Seats shall comply with 610. If a seat is provided, the top of bathtub seats shall be 17 inches minimum and 19 inches maximum above the bathroom finish floor. |  |  |  |  |  |  |  |  |  |  |  |
| 2010 ADA 607.4.2 |  | (33) Grab Bar for Bathtubs Without Permanent Seats. For bathtubs without permanent seats, grab bars shall comply with 607.4.2. Unless otherwise stated grab bars should be mounted 33-36” high from the finished floor. Back Wall. Two grab bars shall be installed on the back wall, one located in accordance with 609.4 and other located 8 inches (205 mm) minimum and 10 inches (255 mm) maximum above the rim of the bathtub. Each grab bar shall be 24 inches (610 mm) long minimum and shall be installed 24 inches (610 mm) maximum from the head end wall and 12 inches (305 mm) maximum from the control end wall. Control End Wall. A grab bar 24 inches (610 mm) long minimum shall be installed on the control end wall at the front edge of the bathtub. Head End Wall. A grab bar 12 inches (305 mm) long minimum shall be installed on the head end wall at the front edge of the bathtub. |  |  |  |  |  |  |  |  |  |  |
| 2010 ADA 607.4.1 |  | (34) Grab Bar for Bathtubs With Permanent Seats. For bathtubs with permanent seats, grab bars shall be provided in accordance with 607.4.1. Unless otherwise stated grab bars should be mounted 33-36” high from the finished floor. Back Wall. Two grab bars shall be installed on the back wall, one located in accordance with 609.4 and the other located 8 inches (205 mm) minimum and 10 inches (255 mm) maximum above the rim of the bathtub. Each grab bar shall be installed 15 inches (380 mm) maximum from the head end wall and 12 inches (305 mm) maximum from the control end wall. Control End Wall. A grab bar 24 inches (610 mm) long minimum shall be installed on the control end wall at the front edge of the bathtub. |  |  |  |  |  |  |  |  |  |  |
| 2010 ADA 607.4.1 |  | (35) Bathtub Controls. Controls, other than drain stoppers, shall be located on an end wall. Controls shall be between the bathtub rim and grab bar, and between the open side of the bathtub and the centerline of the width of the bathtub. Controls shall comply with 309.4 |  |  |  |  |  |  |  |  |  |  |
| 2010 ADA 608 |  | (36) Facilities with more than 50 beds- covered by this part [of the title III regulation or to this section of the title II regulation] that provide common use bathing facilities shall provide at least one roll-in shower with a seat that complies with the relevant provisions of section 608 of the 2010 Standards. Transfer-type showers are not permitted in lieu of a roll-in shower with a seat, and the exceptions in sections 608.3 and 608.4 for residential dwelling units are not permitted. When separate shower facilities are provided for men and for women, at least one roll-in shower shall be provided for each group. Does the shelter have more than 50 Beds? |  |  |  |  |  |  |  |  |  |  |
| 2010 ADA 608.2.1 |  | (37) Transfer Type Shower Compartments. Transfer type shower compartments shall be 36 inches by 36 inches clear inside dimensions measured at the center points of opposing sides and shall have a 36 inch wide minimum entry on the face of the shower compartment. Clearance of 36 inches wide minimum by 48 inches long minimum measured from the control wall shall be provided. Is the compartment the correct size? |  |  |  |  |  |  |  |  |  |  |
| 2010 ADA 608.3.1 |  | (38) Transfer Type Shower Compartments Grab Bars. In transfer type compartments, grab bars shall be provided across the control wall and back wall to a point 18 inches (455 mm) from the control wall. Are the grab bars installed correctly? Unless otherwise stated grab bars should be mounted 33-36” high from the finished floor. |  |  |  |  |  |  |  |  |  |  |

| E. (cont’d) Accessibility – Common Use Restroom |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| “No” items Must have photos. Give actual measurements in comments. “Inc.” means incomplete. “N/A” answers require note in the Comments describing the reason the item is not applicable |  | Restroom # _____ |  |  |  |  | Restroom # _____ |  |  |  |
| Common Use Restroom Only one of each is needed in multiple-lavatory and multiple-commode restrooms |  | Yes | Inc | No | N/A |  | Yes | Inc | No | N/A |
| 2010 ADA 608.2.1 | (39) Transfer Type Shower Compartments Seats. A folding or non-folding seat shall be provided in transfer type shower compartments. A folding seat shall be provided in roll-in type showers required in transient lodging guest rooms with mobility features complying with 806.2. Seats shall comply with 610. The top of seats shall be 17 inches (430 mm) minimum and 19 inches (485 mm) maximum above the bathroom finish floor. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 608.2.1 | (40) Transfer Type Shower Compartments Controls. In transfer type shower compartments, the controls, faucets, and shower spray unit shall be installed on the side wall opposite the seat 38 inches (965 mm) minimum and 48 inches (1220 mm) maximum above the shower floor and shall be located on the control wall 15 inches (380 mm) maximum from the centerline of the seat toward the shower opening. Are the controls in the proper location? |  |  |  |  |  |  |  |  |  |
| 2010 ADA 608.2.2 | (41) Standard Roll-In Type Shower Compartments. Standard roll-in type shower compartments shall be 30 inches (760 mm) wide minimum by 60 inches (1525 mm) deep minimum clear inside dimensions measured at center points of opposing sides and shall have a 60 inches (1525 mm) wide minimum entry on the face of the shower compartment. Is the compartment the correct size? |  |  |  |  |  |  |  |  |  |
| 2010 ADA 608.2.2 | (42) Standard Roll-In Type Shower Compartments. Grab Bars. Where a seat is provided in standard roll-in type shower compartments, grab bars shall be provided on the back wall and the side wall opposite the seat. Grab bars shall not be provided above the seat. Where a seat is not provided in standard roll-in type shower compartments, grab bars shall be provided on three walls. Grab bars shall be installed 6 inches (150 mm) maximum from adjacent walls. Are the grab bars installed correctly? Unless otherwise stated grab bars should be mounted 33-36” high from the finished floor. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 608.2.2 | (43) Standard Roll-In Type Shower Compartments. Controls. In standard roll-in type shower compartments, the controls, faucets, and shower spray unit shall be located above the grab bar, but no higher than 48 inches (1220 mm) above the shower floor. Where a seat is provided, the controls, faucets, and shower spray unit shall be installed on the back wall adjacent to the seat wall and shall be located 27 inches (685 mm) maximum from the seat wall. Are the controls in the proper location? |  |  |  |  |  |  |  |  |  |
| 2010 ADA 608.2.3 | (44) Alternate Roll-In Type Shower Compartments. Alternate roll-in type shower compartments shall be 36 inches (915 mm) wide and 60 inches (1525 mm) deep minimum clear inside dimensions measured at center points of opposing sides. A 36 inch (915 mm) wide minimum entry shall be provided at one end of the long side of the compartment. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 608.2.2 | (45) Shower Spray Unit and Water. A shower spray unit with a hose 59 inches (1500 mm) long minimum that can be used both as a fixed-position shower head and as a hand-held shower shall be provided. The shower spray unit shall have an on/off control with a non-positive shut-off. If an adjustable-height shower head on a vertical bar is used, the bar shall be installed so as not to obstruct the use of grab bars. Shower spray units shall deliver water that is 120°F (49°C) maximum. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 608.2.2 | (46) Shower Thresholds. Thresholds in roll-in type shower compartments shall be ½ inch (13 mm) high maximum in accordance with 303. In transfer type shower compartments, thresholds ½ inch (13 mm) high maximum shall be beveled, rounded, or vertical. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 608.2.2 | (47) Shower Clearance. A 30 inch (760 mm) wide minimum by 60 inch (1525 mm) long minimum clearance shall be provided adjacent to the open face of the shower compartment. |  |  |  |  |  |  |  |  |  |

E. (cont’d) Accessibility – Common Use Facilities

| Common Use Facilities Inspect two if the same type of accessible facility is in more than one location |  | Yes | Inc | No | N/A |  | Yes | Inc | No | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | # 1: |  | _____ |  |  | # 2: |  | _____ |  |
| 2010 ADA 806 | (48) A. Sleeping Rooms - At least one sleeping area shall provide a clear floor space complying with section 305 on both sides of a bed. The clear floor space shall be positioned for parallel approach to the side of the bed. The clear floor or ground space shall be 30 inches minimum by 48 inches minimum. |  |  |  |  |  |  |  |  |  |
|  | (48) B. In sleeping rooms with more than 25 beds covered by this part [of the title III regulation or to this section of the title II regulation], a minimum of 5% of the beds shall have clear floor space complying with section 806.2.3 of the 2010 Standards. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 308.1 | (49) Trash disposal facilities provide at least one accessible trash container or trash chute on an accessible route with hardware not requiring tight grasping pinching, or twisting of the wrist; has a horizontal sliding door w/sill height of max. 48”, or a max. 48” lid height (accessible ramp & landing, or specially marked accessible trash container provided if needed). |  |  |  |  |  |  |  |  |  |
| 2010 ADA 308.1;611.4 | (50) Common-use laundry facilities. At least one laundry room provides at least one accessible washing machine and clothes dryer. Top loading machines shall have the door to the laundry compartment located 36 inches (915 mm) maximum above the finish floor. Front loading machines shall have the bottom of the opening to the laundry compartment located 15 inches (380 mm) minimum and 36 inches (915 mm) maximum above the finish floor. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 308.1 | (51) Mail boxes. Out-going mail slot, parcel locker, rent collection slot, mailbox key lock for each mobility accessible unit, and key locks for a sufficient number of units are on an accessible route and are not higher than 48” |  |  |  |  |  |  |  |  |  |
| 2010 ADA 226.1 | (52) Reception Areas If there is a built in reception area or other type of service counter is there a section that is 36” long and no higher than 36”? |  |  |  |  |  |  |  |  |  |
| 2010 ADA 703.4 | (53) Signage - Do all facility signs include Braille are mounted at the latch side of the door and between 48”- 60” high. |  |  |  |  |  |  |  |  |  |
| 2010 ADA 217 | (54)a. Phones - Is there an accessible phone of each type available? Does it have volume control? |  |  |  |  |  |  |  |  |  |
|  | (54)b. If three or more phones serve one bank is there a shelf and outlet for TTY operation; if four or more phones are provided at a site is there a TTY phone available? |  |  |  |  |  |  |  |  |  |
| 2010 ADA 602.1 | (55) Drinking Fountains – Are all the following requirements confirmed? 1. At least 50% of the drinking fountains must be accessible and on an accessible route. 2. Is adequate clear floor space (30”x 48”) available? 3. Is the top of the spout no higher than 36”? 4. Are the controls on the front of the unit and is the stream at least 4” high? |  |  |  |  |  |  |  |  |  |

| Accessibility (Common Use Facilities) Comments and Observations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Item # |  | Actual Measurement |  |  |  |  | Location/Issue |  |  | Comments (N/A requires comment) |  |

| F. ACCESSIBILITY – Route & Parkin |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| “No” items Must have photos. Give actual measurements in comments. Comment why if N/A. “Inc.” means incomplete. |  | Yes | Inc | No | N/A | Comments |
| Accessible Route in Sleeping Areas, Eating Areas and other Common-use Facilities including Site. |  |  |  |  |  |  |
| 2010 ADA 403 | (52) Route. Min. one accessible route within the boundary of the site is a continuous, safe, and unobstructed path (explain if not safe). Surface is firm, stable, slip resistant material such as pavement, or compact crushed granite. The route must connect parking areas, drop-off areas and all service areas including sleeping, eating, restroom and service provision areas. |  |  |  |  |  |
| 2010 ADA 403 | (53) Route Change in level Along the accessible route connecting all service areas and or amenities is there no change in level greater than ½” (changes in level greater than ¼” but less than ½” should be beveled 1:20 |  |  |  |  |  |
| 2010 ADA 403 | (54) Amenity access. The accessible route provides access to at least one of each type of amenity including sleeping areas, eating areas, supply distribution areas and service areas. |  |  |  |  |  |
| 2010 ADA 403;502.1 | (55) Public access (site arrival points). The accessible route provides access to parking lots w/ accessible parking and at least one public street, public sidewalk, and a public transportation stop (when provided). |  |  |  |  |  |
| 2010 ADA 407 | (56) Elevators. If an elevator or lift provides the only accessible route is there a source of back-up power to operate the device for an extended period. |  |  |  |  |  |
| 2010 ADA 403.5.1 | (57) Route width. Accessible route has min. 36” clear width (not reduced by vehicle overhang). May reduce to min. 32” by an obstruction if the obstruction is no more than 24” wide (Fig. 10). |  |  |  |  |  |
| 2010 ADA 307.1 | (58) Low-headroom and protruding object barrier. Along any walkway, a barrier detectable by a cane is provided directly below the leading edge* of objects protruding more than 4” from a wall (12” from a post) and with bottom of the object between 27” to 80” high (payphones, drinking fountains), and directly below overhead obstructions less than 80” above the walking surface (tree branches, stair structures). Cane detectable barrier is max. 27” high at the bottom or it extends to the walking surface (Fig. 6, 8). |  |  |  |  |  |
| 2010 ADA 403.3 | (59) Running slope along an accessible route (in the direction of travel) is no more than 8.33% (1 in 12) at any point along the route (excl. curb ramps). |  |  |  |  |  |
| 2010 ADA 403.3 | (60) Cross slope along an accessible route (across the direction of travel) is no more than 2% at any point along the route (excl. curb ramps). |  |  |  |  |  |
| 2010 ADA 405.1 | (61) Ramp handrails are provided along the accessible route on both sides of all sections that are sloped more than 5% (ramp), and where the ramp rises over 6 inches or the ramp’s horizontal run is over 6 feet (excl. curb ramps). Rails should be mounted between 34-38”. |  |  |  |  |  |
| 2010 ADA 504.1 | (62) Stairs (min. 2 steps in series) that are nearby an accessible ramp (over 5% slope) connecting levels not connected by an elevator are accessible w/ closed risers, uniform heights (riser to riser), uniform tread depths max. 11”. |  |  |  |  |  |
| 2010 ADA 504.1 | (63) Stair handrails are on both sides of accessible stairs. |  |  |  |  |  |
| Accessible Parking |  |  |  |  |  |  |
| 2010 ADA 502.1 | (64) Number of accessible parking spaces (if parking is provided) should be based on the number of total parking spaces. |  |  |  |  |  |
|  | - If 1-25 spaces are provide-1 Van accessible space (min. width 96” wide) |  |  |  |  |  |
|  | - If 26-50 spaces are provide-1 Van accessible space (min. width of access aisle 96”) and 1 space (min. width of access aisle 60”). |  |  |  |  |  |
|  | - If 51-75 spaces are provide-1 Van accessible space (min. width 96” wide) and 2 spaces (min. width of access aisle 60”). |  |  |  |  |  |
|  | - If 76-100 spaces are provide-1 Van accessible space (min. width 96” wide) and 3 space (min. width of access aisle 60”) |  |  |  |  |  |
|  | - If 101-150 spaces are provide-1 Van accessible space (min. width 96” wide) and 4 space (min. width of access aisle 60”) |  |  |  |  |  |
| 2010 ADA 502.3 | (65) Accessible parking sign w/ the accessibility symbol showing the space is reserved for accessible parking. Sign is high enough not to be obscured by a vehicle parked in the space. |  |  |  |  |  |
| 2010 ADA 502.3 | (66) All Accessible parking spaces are a min. 96” wide (line center-to-center), is completely level (max. 2% slope) and unobstructed. |  |  |  |  |  |
| 2010 ADA 502.3 | (67) Van accessible parking spaces For van-accessible spaces is there vertical clearance of at least 98” |  |  |  |  |  |
| 2010 ADA 502.3 | (68) Access aisle on the accessible route provides access to the route directly off the aisle (w/ curb-ramp as needed) or passes behind only the user’s parking space to provide access to the route on the opposite side. |  |  |  |  |  |

Comments and Observations

| Signature of Inspector: |  | Date: |  |
| --- | --- | --- | --- |
| Signature of Inspection contact: |  | Date: |  |