|  |  | TDHCA Development Inspection Checklist |  |  |  |  | CMTS # |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Italicized item numbers illustrated |  | H. Accessible Site & Common Use |  |  |  |  |  |
| Pictures show context, ‘No’ items and measurements |  |  | Yes | No | N/A | Comments |  |
| Route Connections (check Yes if planned route meets conditions of this section) |  |  |  |  |  |  |  |
| TDHCA/HUD ADA 206.2.1 UFAS 4.3.2 FHA 1.8 | (H-1) Site arrival Min 1 access. route w/in boundaries of the site connects (w/no step if no ramp) all public streets/sidewalks/transportation stops, access. parking/loading zones, and access. bldg/facility entrances they serve. |  |  |  |  |  |  |
| TDHCA/HUD ADA 206.3, 206.2.2 UFAS 4.3.2 FHA 1.8, 2.15 | (H-2) Within site Min 1 access. route (along/near gen circul./ADA) (excl vehicle routes) connects (w/no step if no ramp) access. bldg entrances to: amenities (all, excl Spaces & Elements Section below/ADA) (1/type/UFAS) (1/type/each covered unit building group/FHA); employee work areas. |  |  |  |  |  |  |
| ADA 206.4 UFAS 4.14 FHA 1.10 | (H-3) Building entrance Min 1 access route connect (w/no step if no ramp) common bldg/facility entrances used by all residents/guests/employees and primary entrances of mobility accessible and FHA-covered units. |  |  |  |  |  |  |
| TDHCA/HUD ADA 206.2.3 UFAS 4.3.2 FHA 1.8, 2.15 | (H-4) Within buildings Min 1 access. route (gen circul./ADA) connects (w/no step if no ramp/lift) accessible bldg entrance to: amenities (all, excl Spaces & Elements section below/ADA) (1/type/UFAS/FHA); employee wk areas (excl inaccessible floors if same amenity is access. in the same bldg.) |  |  |  |  |  |  |
| Routes & Circulation Paths |  |  |  |  |  |  |  |
| ADA 403 UFAS 4.5 | (H-5) Surface along access. route and access. rooms/spaces is firm, stable, slip resistant; carpet height max ½” measured to firm backing/pad or floor. |  |  |  |  |  |  |
| ADA 403.3 UFAS 4.3.7 | (H-6) Running/cross slope All points along an access. route provide a running slope <5% (also Ramp Slope below) & a cross slope <2% |  |  |  |  |  |  |
| ADA 303 UFAS 4.3.8 | (H-7) Level changes Steps of over ¼” have beveled edge w/ <1:2 (50%) slope (no-step entries, see Threshold Height below); any change over ½” provides a lift/elevator/ramp (see Resident Elevators & Ramp Slope below). |  |  |  |  |  |  |
| ADA 403 UFAS 4.3.3 | (H-8) Route width is >36W; any reduced-width is >32Wx<24L (no vehicle bumper overhang); passing space is >60x60 every 200 ft. |  |  |  |  |  |  |
| ADA 305 UFAS 4.2.4 | (H-9) CFS Required clear floor or ground space (typ. 30x48) adjoins access. route, has no change in level (<2% slope, see Surface above). |  |  |  |  |  |  |
| ADA 304.3, UFAS 4.2.3, 4.22.3 | (H-10) Turning space All access. spaces w/ no pass-through have >60 dia. or T-turn (base of T >36Wx>24D, arms of T >36Wx>12D; turning space may extend under counter 1 side only, see Knee & Toe below) (>30x60 unobstructed turning space in toilet rooms having 1 toilet, 1 lav OK/UFAS). |  |  |  |  |  |  |
| ADA 306 UFAS 4.15.5, 4.19.2, 4.32.3 | (H-11) Knee & toe Any clearance required as part of a CFS beneath or adjacent to an element is >30W (>36W if part of a T-turn) and is open, min clear depth at knee is 8D at 27H; at ankle is 11D at 9H; at toe is 17D. |  |  |  |  |  |  |
| ADA 307.2, 4 UFAS 4.4.2, | (H-12) Overhead hazard Circul. paths/adjacent areas have barriers beneath where clearance is <80H; any cane detectable guardrail barrier is <27H. |  |  |  |  |  |  |
| ADA 307 UFAS 4.4.1 FHA 2.18 | (H-13) Protruding objects Pedestrian circulation paths provide a barrier beneath the leading edge of any object that protrudes more than 4” from a wall (12” from a post) if the bottom of the leading edge of the object >27H. |  |  |  |  |  |  |
| ADA 405-406 UFAS 4.7.2, 4.8.2 FHA 1.7, 2.17 | (H-14) Ramp slope Ramps, curb ramps along access. rt. (near stairs/FHA) 5% to 8.33% slope (<5% N/A), max 30H rise (rehabs w/ space limitations 10% to 12.5% slope, max <3H rise, or 8.33% to 10% slope, max 6H rise). |  |  |  |  |  |  |
| ADA 405.7 UFAS 4.8.4 FHA 1.7 | (H-15) Ramp landing top/bottom is level (<2%); >60L by ramp-width wide; >60x60 landing at 2-ramp change in direction (curb ramp N/A). |  |  |  |  |  |  |
| ADA 406 UFAS 4.7 FHA 1.7 | (H-16) Curb ramp side-flares w/ cross traffic have <10% slopes plus a detour around the top of ramp (>36W/ADA) (>48W/UFAS/FHA) (in rehab where min width of the detour is not possible, flared side slopes are <8.33%/ADA); gutter counter-slope at bottom is <5% w/ smooth transition. |  |  |  |  |  |  |
| ADA 405.9 UFAS 4.8.7 | (H-17) Ramp edge Ramps/landings w/ a drop at the edge have a barrier rail/curb (stops 4” sphere/ADA) (>2H/UFAS) or >12” projecting surface. |  |  |  |  |  |  |
| ADA 210.1 UFAS 4.1.2(4) FHA 1.7, 2.17 | (H-18) Stairs Int/ext stairs (for egress rehab w/ no lift/ramp, and egress new/ADA) (in bldgs w/no ramp/lift new/UFAS) (egress stairs & stairs near accessible ramp w/no lift between levels/FHA) provide uniform closed risers 4-7H; uniform treads >11D; any curved nosing <½” radius, any nosings projecting beyond risers (<1½”) have curved/beveled underside. |  |  |  |  |  |  |
| ADA 405.8, 505 UFAS 4.8.5, 4.9.4 FHA 1.7 | (H-19) Handrails both sides of access. stair or ramp w/ >6H rise (or >72L run/UFAS/FHA); 34-38H from top of rail to stair nosing or ramp surface; cross-sect. 1¼” min to (2”/ADA) (1½”/UFAS) max dia.; >1½” clearance around rail (support bracket >1½L OK) (no bracket at top/sides/ADA). |  |  |  |  |  |  |
| ADA 405.8, 505.10 UFAS 4.8.5, 4.9.4 FHA 1.7 | (H-20) Handrail extensions (N/A in rehab if hazardous) are level >12” past top & bottom of ramps, past top of stairs; >12” past bottom riser at slope of stairs (plus an additional 1 tread-depth at level/UFAS) (extends in same direction as stair or ramp/ADA); ends return to floor/wall/post. |  |  |  |  |  |  |
| ADA 206.6 408 UFAS 4.3, 4.10 | (H-21) Resident elevators Door sill level +½”; open >3 seconds, reopens if obstructed; call button (in reach range or <54H in rehab/ADA) (42H/UFAS); car controls in reach range (<54H rehab OK); emergency controls incl alarm are grouped at bottom of the panel >35H. |  |  |  |  |  |  |

| Pictures show context, ‘No’ items and measurements |  |  |  |  |  | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Accessible Parking |  |  | a + b + c |  |  |  |  |  |  |
| ADA 208.2.3 UFAS 4.1.1 FHA 2.23 |  | (H-22) Leasing & public parking areas (each area if separate) have >1 access. space per 25 parking spaces (excl b & c spaces). |  | a. |  |  |  |  |  |
| ADA 208.2.3 UFAS 4.1.1 |  | (H-23) Mobility units Where each unit has min 1 space, each mobility unit has min 1 accessible space (excl a & c spaces). |  | b. |  |  |  |  |  |
| ADA 208.2.3.2 UFAS 4.1.1 FHA 2.23-24 |  | (H-24) Residents & visitors Access. spaces are the greater of: 2% of spaces over 1 space per unit/ADA/UFAS (excl a & b), or 2% of spaces serving FHA units/FHA (excl a if opposite gate). |  | c. |  |  |  |  |  |
| ADA 208.2.4, 502.5 |  | (H-25) Van parking Min 1 space/6 access. spaces (round up); space/aisle/vehicle route >98H (see Access Space, Aisle below). |  |  |  |  |  |  |  |
| FHA 2.23 |  | (H-26) Amenities Any parking near amenities provides min 1 accessible space (may incl c, excl b, excl any a spaces opposite control gate). |  |  |  |  |  |  |  |
| ADA 208.3.2 UFAS 4.6.2 FHA 2.20 |  | (H-27) Shortest route Spaces for leasing office, mobility units (a & b), and FHA-covered units are on shortest access. route to the entrance they serve. |  |  |  |  |  |  |  |
| ADA 208.3.2 FHA 2.23 |  | (H-28) Dispersion Mobility & resident spaces (b & c above) are dispersed among types provided (surface/garage/carport) (min 1 ea type/FHA). |  |  |  |  |  |  |  |
| ADA 502.2 UFAS 4.6.3 FHA 2.20 |  | (H-29) Parking space Car space >96W, (van >132W or >96W w/ >96W access aisle/ADA) (van >96W/UFAS); level <1:48–2% (see Surface above). |  |  |  |  |  |  |  |
| ADA 502.3 UFAS 4.6.3 FHA 2.20 |  | (H-30) Access aisle is >60W (van >96W/UFAS), on an access. route (curb-cut or flush/top of curb); no obstr between aisle & parking space; level (<2% slope OK) (marked/ADA), no built-up ramp (see Surface above) |  |  |  |  |  |  |  |
| Signage | Tactile–Raised character, standard font, sans serif, 5/8–2H, upper case w/braille Visual–High contrast character, standard font, >5/8 H, upper/lower case, no glare |  |  |  |  |  |  |  |  |
| ADA 216.2, 216.4.1, 703 |  | (H-31) Room/egress/unit Any sign identifying a permanent int/ext room, space, or egress route has both Tactile & Visual characters (excl Tactile for ext rm/space signs not located at a door (see Room/Egress Location below). |  |  |  |  |  |  |  |
| ADA 216.2, 703.4 |  | (H-32) Room/egress/unit sign location is on push-side of a door face w/ closer (incl self-close hinge, excl hold-open device), or on wall at latch-side; CFS is 18x18 centered on the raised characters, lowest row baseline >48H, highest is <60H (sign center <60H in pre-2012 rehab if no CFS obstruction). |  |  |  |  |  |  |  |
| ADA 216.2, 703.6.1 |  | (H-33) Pictograms Any pictorial symbol identifying a permanent interior room/space is high contrast on a blank >6H field; has Visual & Tactile text descriptors below it (excl Tactile text for ext room/space if not at a door). |  |  |  |  |  |  |  |
| ADA 216.3, 703.4-5 |  | (H-34) Directional/informational Signs to/about int spaces/facilities (e.g. “office hours”) are Visual w/ character baseline >40H, or >48H if raised characters/braille (i.e. Room/Egress Location, above) (excl elevators). |  |  |  |  |  |  |  |
| ADA 216.6, 216.8 |  | (H-35) Entrances & restrooms Accessible entrances/restrooms provide a sign w/ chair symbol; inaccessible entrances/restrooms (i.e. infeasible rehab) have a Directional/Informational sign (above) w/ chair symbol indicating location of nearest accessible facilities. |  |  |  |  |  |  |  |
| UFAS 4.30.5 |  | (H-36) Entrances & restrooms have sign, wheelchair symbol if accessible. |  |  |  |  |  |  |  |
| ADA 407 UFAS 4.10 |  | (H-37) Elevator signals Visible signals indicate when call & car controls are registered & answered, visible travel direction is at hall entrance >72H (no visible signal in rehab OK/ADA); audible signal once/up, twice/down; audible/visible signals indicate floor passed, floor stopped. |  |  |  |  |  |  |  |
| ADA 407 UFAS 4.10 |  | (H-38) Elevator designations Car controls are labeled w/ Tactile characters for each floor level (main floor w/ star); symbols indicate emergency stop, alarm, door open/close, phone; Tactile signs are on both door jambs at each floor, 48-60H, with >2H numbers (main floor w/ star). |  |  |  |  |  |  |  |
| ADA 216.5 UFAS 4.6.4 FHA 2.20 |  | (H-39) Parking sign Wheelchair symbol identifies access. parking/loading zones (>60H bottom of sign to parking surface/ADA) (not obscured by vehicle/UFAS) (van space parking sign says “Van Accessible”/ADA). |  |  |  |  |  |  |  |
| Doors & Gates (passage doors on accessible route) |  |  |  |  |  |  |  |  |  |
| ADA 404.2.3, 404.2.9 UFAS 4.13.5 |  | (H-40) Clear opening >32W (313/8 rehab OK) door stop to door face at 90°. 5 lbs force maximum for opening. |  |  |  |  |  |  |  |
| ADA 404.2.4 UFAS 4.13.6 |  | (H-41) Maneuvering space Clear/level (<2% slope) both sides of entry/passage doors within required CFS per appr condition: (a) front appr/pull >18W beyond latch, (b) latch or hinge appr/push 42W hall, (c) latch appr/pull 48W hall (d) hinge appr/pull 54W hall w/ 42W beyond latch. |  |  |  |  |  |  |  |
| ADA 303, 404.2.5 UFAS 4.3.8, 4.13.8 |  | (H-42) Threshold height plus any floor level change is <½” (¾”rehab OK) from top of threshold to floor/landing surface (<¾” at sliding door/UFAS); provides a beveled edge 1:2 slope (50%) at any step in level over ¼”. |  |  |  |  |  |  |  |
| ADA 404.2.7 |  | (H-43) Pool gate hardware Operable parts of the release of latch on any self-latching devices <54H (34-48H if device also self-locking, see above). |  |  |  |  |  |  |  |
| ADA 404.2.10 |  | (H-44) Kick plate Smooth surface up to 10” from the walking surface is provided on the push-side of new swinging door/gate, extending full width; cavities (e.g. between bars, joints) created by an added kick plate are capped |  |  |  |  |  |  |  |

| Pictures show context, ‘No’ items and measurements |  |  |  | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reach Ranges & Operating Mechanisms |  | Obstructed forward | <48H reach w/ <20D obstr Or <44H w/ 20-25D obstr (e.g. Dispenser above lav 48H, above counter 44H/ADA/UFAS) |  |  |  |  |
|  |  | Unobstructed forward | 15-48H reach (e.g. Wall-mounted telephone in a corner 48H/ADA/UFAS) |  |  |  |  |
|  |  | Obstructed side | <46H reach w/ <34Hx10-24D obstruction (e.g. Outlet or switch above a low counter 46H/ADA/UFAS) |  |  |  |  |
|  |  | Unobstructed side | (15-48H/ADA) (9-54H/UFAS) w/ <34Hx<10D obstr (e.g. Mailbox above protruding wainscot 48H/ADA, 54H/UFAS) |  |  |  |  |
| ADA 205, 308-309 UFAS 4.2, 4.27 | (H-45) Operable parts Door/cabinet/faucet handles, hardware, dispensers (exercise mach. N/A) (no push-type, thumb-latch/ADA) on an access. route; require 1 hand, no tight grasping, pinching, twisting wrist; in reach range. |  |  |  |  |  |  |
| ADA 205, 308-309 UFAS 4.2, 4.27 | (H-46) Controls & outlets All switches, controls (no recessed buttons) (excl staff-only use), outlets (excl dedicated outlets) (min 1 per area/UFAS) on an access. route; in reach range. |  |  |  |  |  |  |
| ADA 205,308-9 UFAS 4.2,4.27 | (H-47) Mailboxes Rent deposit slot, mobility unit key lock, and min 1 mail out-slot are on an access. route; in reach range. |  |  |  |  |  |  |
| ADA 308-9 UFAS 4.2,4.27 FHA 2.16 | (H-48) Trash disposal Container/dumpster/chute (number/location see Within Site above) on access. rt.; opening/sill/operable parts in reach range. |  |  |  |  |  |  |
| TDHCA/HUD ADA 214,611 4.1.3,4.34.7.2 | (H-49) Laundry Min 1 front-loading washer/dryer (incl every laundry rm, excl inaccessible floors if accessible laundry is in same bldg) are on accessible route; door in reach rage. |  |  |  |  |  |  |
| Kitchens, Grills & Sinks (indicate location of ‘No’ items) |  |  |  |  |  |  |  |
| ADA 804.2.1 FHAG 7 | (H-50) Pass through (i.e. entry at both ends) >40W between opposing surfaces excl handles (N/A if no cooktop or range, e.g. some SROs/ADA). |  |  |  |  |  |  |
| ADA 804.2.2 | (H-51) Non-pass through (i.e. entry at 1 end) >60W between opposing surfaces excl handles (N/A if no cooktop or range, e.g. some SROs/ADA). |  |  |  |  |  |  |
| FHAG 7 | (H-52) Non-pass through >60” turning circle if a range, cooktop or sink is at the end; >40W clearance if no range/cooktop/sink, or open beneath. |  |  |  |  |  |  |
| FHAG 7 | (H-53) Appliance/grill CFS Side appr at range, cooktop, sink, grill (any front-only appr has protection beneath, see Knee & Toe above); side or front at refrigerator, dishwasher, oven. |  |  |  |  |  |  |
| ADA 606, 804 UFAS 4.24 FHAG 7 | (H-54) Sink is <34Hx>30W w/ >30W open clear space beneath (clearance at min 1 bowl if multi-bowl/ADA) (see Knee & Toe above); CFS positioned for front approach (centered side approach OK if no cooktop or range, e.g. some SROs/ADA; no overlapping dishwasher door/ADA). |  |  |  |  |  |  |
| ADA 606.4,5 UFAS 4.24.6 | (H-55) Pipes are protected against contact, no sharp or abrasive surface. |  |  |  |  |  |  |
| ADA 804.6.4 | (H-56) Range controls require no reach across burners. |  |  |  |  |  |  |
| ADA 804.6.6 | (H-57) Refrigerator/freezer has >50% of the freezer space <54H. |  |  |  |  |  |  |
| Restrooms (indicate location of ‘No’ items) |  |  |  |  |  |  |  |
| ADA 603.2.3 UFAS 4.22.2 | (H-58) Door swing No passage door swings into the CFS required for any fixture (excl single-use if 30x48 maneuvering is outside door swing/ADA). |  |  |  |  |  |  |
| ADA 604.2 | (H-59) Toilet & placement Min 1 in corner of room or stall 16-18” center to sidewall or partition; 17-19H seat; flush control on open side of toilet. |  |  |  |  |  |  |
| ADA 604.3 | (H-60) Toilet CFS is >60Wx56D w/no overlapping fixture or obstructions other than dispensers, disposal units, coat hooks, shelves, GB. |  |  |  |  |  |  |
| UFAS 4.16.2 FHA 2.28 | (H-61) Toilet CFS If front approach is possible, toilet CFS is >48Wx66D or >60Wx56D; side-only is >48Wx56D; edge of any overlapping wall-hung lav is >18” to toilet centerline. |  |  |  |  |  |  |
| ADA 213.3, 604.8 UFAS 4.17 | (H-62) Stall CFS Where stalls are provided, min 1 is wheelchair w/ CFS >60Wx56D for wall-hung, or >60Wx59D for floor-mounted; partition bottom is >9H for toe clearance at front partition & one side partition (excl front partition if CFS >6” deeper, side partition if CFS >6” wider than min). |  |  |  |  |  |  |
| ADA 213.3, 604.8 UFAS 4.17 | (H-63) Stall door meets hardware/door/maneuvering (>42W hall OK if pull-side/latch appr); hinge-side of door is <4” from the corner diagonally opposite toilet (self-close hinges, door pulls near latch both sides/ADA). |  |  |  |  |  |  |
| ADA 604.5, 609 UFAS 4.16.4 FHA 2.28 | (H-64) Grab bars Horizontal, 33-36H; 1½” to wall (>1½” to obstr); toilet sidewall GB extends from the corner <12” to >54” (<12” to >52” at stall/UFAS), >36” rear GB extends from toilet centerline to >12” & >24” (centered >24” rear GB OK if adjacent lav is recessed in rear wall/ADA). |  |  |  |  |  |  |
| ADA 604.7 | (H-65) TP dispenser located 7-9” center to front of toilet; outlet 15-48H; wall-dispenser >1½” below GB or >12’’ above; allows continuous flow. |  |  |  |  |  |  |
| ADA 606 UFAS 4.19 FHA 2.28 | (H-66) Lavatory Min 1 w/ rim/counter surface <34H; >30W open clear space beneath; CFS positioned for front appr (see Knee & Toe above). |  |  |  |  |  |  |
| ADA 606.4,5 UFAS 4.24.6 | (H-67) Pipes are protected against contact, no sharp or abrasive surface. |  |  |  |  |  |  |
| ADA 213.3.5 UFAS 4.19.6 | (H-68) Mirror bottom above lav/counter <40H (<35H if not above/ADA). |  |  |  |  |  |  |
| ADA 213.3.7, 603.4 | (H-69) Hooks & shelves Min 1/type provided in reach (shelf 40-48H). |  |  |  |  |  |  |

| Pictures show context, ‘No’ items and measurements |  | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Spaces & Elements |  |  |  |  | #73-80 – Indicate Number of Each Per Area |
| ADA 211, 602 | (H-70) Drinking fountain >50% of total; >1 low, 1 high spout; low <36H, >15” from vertical support, water flow >4H, <5” from & parallel w/ front; high spout 38-43H; CFS centered front appr, open below (see Knee & Toe). |  |  |  |  |
| UFAS 4.1.2(9), 4.15 | (H-71) Drinking fountain min 50% of provided/floor; 1 spout is <36H; water flow >4H, parallel w/ front of fount.; CFS for side appr if free-standing or built-in; if forward appr is open below (see Knee & Toe above). |  |  |  |  |
| ADA 221, 802 UFAS 4.33 | (H-72) Theater seating If fixed seating, has wheelchair parking >36Wx48D (>60D if side-only appr) (min 1 space w/ companion seat if 4-25, 2 spaces if >25 seats, >5% of aisle seats designated reserved w/ retractable aisle-side arm rest located nearest access. route/ADA) (5% of all seats/UFAS). |  |  |  |  |
| ADA 219, 706 | (H-73) Assistive listening Theater rooms provide min 2 are hardwired or wireless system w/ receivers (min 2, hearing-aid compatible). |  |  |  |  |
| ADA 226, 902 UFAS 4.1.2(17), 4.32 FHA 1.8, 2.7 | (H-74) Tables/work surface if provided (min 5%, dispersed/ADA/UFAS) (1/area served/FHA); surface 28-34H; CFS positioned for front appr; >30W open clear space beneath (see Knee & Toe). |  |  |  |  |
| ADA 225.3 UFAS 4.25 | (H-75) Storage on-site Any individual self-storage spaces provide min 5% accessible; dispersed among any different types. |  |  |  |  |
| ADA 236 UFAS4.1.3(3) FHA 2.7 | (H-76) Exercise machines Min 1 ea type of machine has CFS on an access. route positioned for wheelchair transfer or for use from a wheelchair. |  |  |  |  |
| ADA 206.2.12 UFAS 4.1.3(3) FHA 2.7 | (H-77) Sports courts/fields Access. route connects directly to both sides of sports courts and to a boundary of sports fields. |  |  |  |  |
| UFAS 4.1.3(3) FHA 2.7 | (H-78) Pool & playground Min 1 access. route connects to a boundary of children’s play areas; to the edge and around the apron of pools. |  |  |  |  |
| ADA 242.2, 1009 | (H-79) Pool entry Small/med size pool (e.g. <75’x75’) has an access. ramp or lift, or 2 other means of entry (see Other Means below); a large pool has 2 means of entry where at least 1 is a ramp or lift (check Yes if planned means of entry meets pool entry number/type, see Pool Lift & Pool Sloped Entry below). |  |  |  |  |
| ADA 242.3, 1009.3 | (H-80) Wading entry Shallow pool (i.e. children’s play pool, excl tanning shelf) has an accessible sloped entry (see Pool Sloped Entry, excl handrails). |  |  |  |  |
| ADA 1009.2 | (H-81) Pool lift Any accessible pool lift is permanently fixed; located where water is <48D; seat is >16W, 16-19H above deck, centerline >16” from pool edge; with 36x48 CFS positioned opposite the water for side appr that is flush with a line located 12” behind the rear edge of the seat. |  |  |  |  |
| ADA 1009.2 | (H-82) Lift operation Lifting capacity rated for 300 pound load (holding capacity is >1.5 times lifting capacity); footrests move w/ seat; any armrest is opposite the water & removable or folds clear of seat; can call from deck or water when in opposite position; seat submerges >18” below water level. |  |  |  |  |
| ADA 242.2, 1009.3 | (H-83) Pool sloped entry Any accessible sloped entry meets ramp requirements (see Ramp Slope, Landing, and Handrails above – excl slip resistance, rail extensions at bottom landing, handrails at wading pools); bottom landing is 24-30” below water level; handrails are 33-38” apart. |  |  |  |  |
| ADA 242.2, 1009.6 | (H-84) Secondary entry If a transfer wall or transfer steps are used to meet entry requirements (see Pool Entry above), architect’s 2010 ADA compliance certification is provided; if pool stairs are a means of entry, stairs & handrails meet accessibility requirements (see Stairs, Handrails, Handrail Extensions above, excl 4-7H risers, bottom extensions; handrails 20-24” apart) (N/A if lift or ramp are means of entry for small/med pool). |  |  |  |  |

| ADA Accessible Play Area #1 (use separate checklist for each play area) |  |  |  |  |  |  |  |  |  |  |  |  |  | Tots 5-12yr |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Definitions Play Component Element for play/socializing/learning; single or in a composite structure (counted as single component if in a group of identical elements). Ground Level Component has ground level-only approach and exit (e.g. free-standing slide or swing set). Elevated Component is approached from above grade (e.g. platform) or below grade, and is part of a composite play structure with 2 or more components. Accessible Component has an adjacent 30”x48” clear space on an accessible route and complies with applicable checklist items below. Experience Type is the primary general experience provided by the component: 1) spinning 2) swinging (group or single swing) 3) sensory (sound making, viewing place) 4) sliding (spiral, straight, double) 5) object/machine play 6) rocking 7) climbing (e.g. chin-up, monkey bars, crawl tube) 8) pretend/game 9) other: |  |  |  |  |  |  |  |  |  |  |  |  |  | Experience Types (Definitions, from components list below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ground tally: (e.g.: 1, 1, 2, 3, 6, 6, 9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Elevated tally: (e.g.: 4, 4, 4, 5, 6, 9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Component Numbers (count as accessible if planned as accessible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (a) Different types ground component: (“5”/e.g. above) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (b) Accessible ground components: (“7”/e.g., accessible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (c) Different types access. ground comp: ((a) & accessible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (d) Elevated components: (“6”/e.g. above) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (e) Elevated components on transfer syst. (“6”/e.g., access.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (f) Elevated components on ramp: (“N/A”/e.g.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (g) Diff. types elevated components on ramp: (“N/A”/e.g.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pictures show context, ‘No’ items and measurements |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | No | N/A |  | Comments |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.2.6 | (H-85) Ground surface certifications: (1) architect’s ASTM F 1951-99 (force to propel wheelchair across surface) & ASTM F 1292-99/04 (use zone impact surface); (2) owner’s regular/frequent erosion maintenance. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 240.2.1.1 | (H-86) Min. different types of accessible ground components Where ground components are provided, min 1 accessible ground-level component is provided for each of the experience types available among all ground level components (see Experience Types above) (c) = (a). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA Table 240.2.1.2 | (H-87) Additional access. ground components per number of elevated The total number & number of types of access. ground components meets 2010 ADA Table 240.2.1.2 (column 2 & 3) based on the total of elevated access./non-access. components (find (d) above in column 1 of the table): (b) > Column 2 & (c) > Column 3 (N/A if (d) <1 or ½ (d) > (f) and (g) >3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 240.2.2 | (H-88) Min accessible elevated components Where elevated components are provided at least half are accessible ((e) + (f)) > ((50% of (d)). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 240.2.1 | (H-89) Ground distribution Accessible ground level components are dispersed and integrated among components. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 206.2.17, 1008.2.4.1 | (H-90) Ground route connects to 30x48 CFS at access. component entry & exit point, >44W (for small play area, typ. <32’x32’) w/ >60” circle or T-turn every 30 ft., >80” vert. clearance. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.2.5.1, 405.6 | (H-91) Ground ramp if any, has running slope <6.25%, cross slope <2%, <30” rise per ramp w/ level landings >60L by ramp-width wide at top & bottom, 60x60 at any 2-ramp change in direction; 60” circle or T-turn on same level as access. components. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.4 1008.2.4.2 | (H-92) Elevated route (excl transfer platform, steps, below) connects to 30x48 CFS at access. component entry & exit points, >36W w/ any reduced-width segment >32Wx<24L, any narrow segments are >48” apart. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.2.5.2 | (H-93) Elevated ramp if any, has running slope <8.33%, cross slope <2%, <12” rise per ramp w/ level landings >60L by ramp-width wide at top & bottom, 60x60 landing at any two-ramp change in direction; 60” circle or T-turn on same level as access. components. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.3.1 | (H-94) Transfer platform if any, has >14x24 level surface, 11–18H above an adjacent 30x48 wheelchair parking space that is parallel & centered on an unobstructed >24” side of the platform, provides transfer support (e.g. rope loop, pole, horizontal or vertical slot). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.3.2 | (H-95) Transfer steps connect platform w/ access. elevated components, >14Dx24W level surfaces, <8H, transfer supports (standard handrails N/A). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 505, 1008.2.5.3 | (H-96) Ramp handrails if any, are 20–28H above ramp, 1–1½ dia. grip (handrail extensions N/A). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.4.4-5 | (H-97) Entry point/seat for transfer onto a component is 11–24H above wheelchair parking surface (excl slides), provides transfer support. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ground Components |  | A | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  | Elevated Components |  |  |  | A |  | 1 | 2 | 3 |  | 4 | 5 | 6 | 7 | 8 |  | 9 |

| ADA Accessible Play Area #2 (use separate checklist for each play area) |  |  |  |  |  |  |  |  |  |  |  |  |  | Tots 5-12yr |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Definitions Play Component Element for play/socializing/learning; single or in a composite structure (counted as single component if in a group of identical elements). Ground Level Component has ground level-only approach and exit (e.g. free-standing slide or swing set). Elevated Component is approached from above grade (e.g. platform) or below grade, and is part of a composite play structure with 2 or more components. Accessible Component has an adjacent 30”x48” clear space on an accessible route and complies with applicable checklist items below. Experience Type is the primary, general experience provided by the component: 1) spinning 2) swinging (group or single swing) 3) sensory (sound making, viewing place) 4) sliding (spiral, straight, double) 5) object/machine play 6) rocking 7) climbing (e.g. chin-up, monkey bars, crawl tube) 8) pretend/game 9) other: |  |  |  |  |  |  |  |  |  |  |  |  |  | Experience Types (Definitions, from components list below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ground tally: (e.g.: 1, 1, 2, 3, 6, 6, 9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Elevated tally: (e.g.: 4, 4, 4, 5, 6, 9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Component Numbers (count as accessible if planned as accessible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (a) Different types ground component: (“5”/e.g. above) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (b) Accessible ground components: (“7”/e.g., accessible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (c) Different types access. ground comp: ((a) & accessible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (d) Elevated components: (“6”/e.g. above) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (e) Elevated components on transfer syst. (“6”/e.g., access.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (f) Elevated components on ramp: (“N/A”/e.g.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | (g) Diff. types elevated components on ramp: (“N/A”/e.g.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pictures show context, ‘No’ items and measurements |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | No | N/A |  | Comments |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.2.6 | (H-85) Ground surface certifications: (1) architect’s ASTM F 1951-99 (force required to propel wheelchair across surface) & ASTM F 1292-99/04 (use zone impact surface); (2) owner’s regular/frequent erosion maintenance. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 240.2.1.1 | (H-86) Min. different types of accessible ground components Where ground components are provided, min 1 accessible ground-level component is provided for each of the experience types available among all ground level components (see Experience Types above) (c) = (a). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA Table 240.2.1.2 | (H-87) Additional access. ground components per number of elevated The total number & number of types of access. ground components meets 2010 ADA Table 240.2.1.2 (column 2 & 3) based on the total of elevated access./non-access. components (find (d) above in column 1 of the table): (b) > Column 2 & (c) > Column 3 (N/A if (d) <1 or ½ (d) > (f) and (g) >3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 240.2.2 | (H-88) Min accessible elevated components Where elevated components are provided at least half are accessible ((e) + (f)) > ((50% of (d)). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 240.2.1 | (H-89) Ground distribution Accessible ground level components are dispersed and integrated among components. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 206.2.17, 1008.2.4.1 | (H-90) Ground route connects to 30x48 CFS at access. component entry & exit point, >44W (for small play area, typ. <32’x32’) w/ >60” circle or T-turn every 30 ft., >80” vert. clearance. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.2.5.1, 405.6 | (H-91) Ground ramp if any, has running slope <6.25%, cross slope <2%, <30” rise per ramp w/ level landings >60L by ramp-width wide at top & bottom, 60x60 at any 2-ramp change in direction. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.4 1008.2.4.2 | (H-92) Elevated route (excl transfer platform, steps, below) connects to 30x48 CFS at access. component entry & exit points, >36W w/ any reduced-width segment >32Wx<24L, any narrow segments are >48” apart. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.2.5.2 | (H-93) Elevated ramp if any, has running slope <8.33%, cross slope <2%, <12” rise per ramp w/ level landings >60L by ramp-width wide at top & bottom, 60x60 landing at any two-ramp change in direction w/ 60” circle or T-turn on same level as access. components. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.3.1 | (H-94) Transfer platform if any, has >14x24 level surface, 11–18H above an adjacent 30x48 wheelchair parking space that is parallel & centered on an unobstructed >24” side of the platform, provides transfer support (e.g. rope loop, pole, horizontal or vertical slot). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.3.2 | (H-95) Transfer steps connect platform w/ access. elevated components, >14Dx24W level surfaces, <8H, transfer supports (standard handrails N/A). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 505, 1008.2.5.3 | (H-96) Ramp handrails if any, are 20–28H above ramp, 1–1½ dia. grip (handrail extensions N/A). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADA 1008.4.4-5 | (H-97) Entry point/seat for transfer onto a component is 11–24H above wheelchair parking surface (excl slides), provides transfer support. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ground Components |  | A | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  | Elevated Components |  |  |  | A |  | 1 | 2 | 3 |  | 4 | 5 | 6 | 7 | 8 |  | 9 |